###### Franklin County Subdivision Regulations

Acting pursuant to Section 232.003, Texas Local Government Code, the Commissioners' Court of Franklin County, Texas, hereby adopts the following regulations governing the subdivision of land. These regulations shall be known as the Franklin County Subdivision Regulations; further, to fulfill the obligation mandated upon Franklin County as dictated by state law, in particular, Texas Local Government Code Section 232.003.

- Purpose

  - These regulations have been prepared in general to aid in the orderly development of Franklin County, Texas, and provide guidelines that will lead to a desirable environment. These regulations supersede all previous Franklin County Subdivision Regulations. Specifically, they have been prepared for the following purposes:

1.1.1	To furnish the developer with guidance and assistance in the expedient preparation and approval of his or her plat.

1.1.2	To protect the citizens of Franklin County by ensuring minimum subdivision and development guidelines for residential, commercial and industrial subdivisions.

1.1.3	To provide for the welfare of the public by providing guidelines for the location, design, and construction of roadways, roadway intersections, drainage, water and wastewater improvements, and other features that provide for the safety of the general public.

1.1.4	To prevent the county road system from being burdened with substandard streets or roads in the future.

1.1.5	To regulate on-site sewage facilities and to protect rights of owners of interest in groundwater.

1.1.6	To respond to a real and substantial threat to public health and safety and to significantly advance the health and safety purpose overall, and not to impose a greater burden than is necessary to achieve the health and safety purposes for the general public.

NOW, THEREFORE, BE IT HEREBY ORDERED BY THE COMMISSIONERS’ COURT OF FRANKLIN COUNTY THE FOLLOWING:

###### Definition of Terms

  - Subdivision

Defined by section 232.001, Local Government Code, as 'The division of a tract of land into two or more parts to lay out

- a subdivision of the tract, including an addition;

- lots; or

- streets, alleys, squares, parks, or other parts of the tract intended to be dedicated to public use or for the use of the purchasers or owners of lots fronting on or adjacent of the streets, alleys, squares, parks, or other parts.

The Franklin County Commissioners' Court has adopted guidelines, incorporated in these Regulations as Appendix A, stating when they require owners to file a plat for a subdivision.

  - Street or Road

The terms "street'' or "road" are interchangeable and mean a vehicular way or a way for vehicular traffic and are used to describe all vehicular ways regardless of any other designation they may have. Streets and roads shall be dedicated to the public except as indicated in Section 7.

  - Arterial Streets or Roads

Arterial streets or roads are those that are principally regional in nature and are used for through or heavy traffic and shall be divided into two classifications:

    - Streets or roads that will serve vehicular traffic beyond the limits of the subdivision; and/or connect one collector or arterial with one or more collectors or arterials.

    - Streets or roads that are existing county roads, and carry a numerical designation.

  - Collector streets or roads

Collector streets or roads are those that connect arterial streets or roads with local streets or roads.

  - Local Streets or Roads

Local streets or roads are those that principally provide direct access to lots within a subdivision.

  - Urban Street

For the purposes of this ordinance, an urban street is any street situated such that the spacing of driveways is less than 100 feet apart for a distance of ¼ of a mile. This distance does not apply to a subdivision using curb and gutter. Any curb and gutter street will be considered an urban street.

  - Rural Street

For the purposes of this ordinance, a rural street is any street situated such that the spacing of driveways is greater than 100 feet apart.

  - Major Highway or Road

All roads maintained by the Texas Department of Transportation are considered a major highway or road.

  - Precinct Commissioner

All references in these Regulations to "Precinct Commissioner'' shall be construed to refer to the Franklin County Commissioner in whose precinct the subdivision is located. If the subdivision is located in more than one precinct, approvals affecting the subdivision as a whole shall be obtained from each of the affected Precinct Commissioners.

  - Subdivision

Defined by LGC, Section 232.001, all references in these regulations to “subdivision” shall be construed to refer to the division of a tract of land into two or more parts to lay out: a) subdivision of the tract, including an addition; b) lots, or c) streets, alleys, squares, parks or other parts of the tract intended to be dedicated to public use or for the use of purchasers/owners of lots fronting on or adjacent to the streets, alleys, squares, parks, or other parts.

Minimum Requirement

Requirements, when defined as minimum, shall be the minimum acceptable requirements. The county, due to issues pertaining to each unique subdivision, may increase such requirements.

  - Owner

All references in these Regulations to an "Owner" shall be construed to refer to the person or persons possessing title and/or lien to the property within the subdivision. This can also refer to the owner's surveyor, engineer, attorney, or planner who has been given written authority to represent the owner.

  - Developer

Persons, corporations, organizations, governmental or governmental subdivisions or agencies, estates, trusts, partnerships, associates, incorporations, or other entities that undertake the activities covered by these regulations.

  - Preliminary Plat

A map or drawing of a proposed subdivision meeting the requirements of Section 3.2.

  - Final Plat

A map or drawing of a proposed subdivision prepared in a manner suitable for recording in the County records and prepared in conformance with the conditions of preliminary approval previously granted by the Commissioners' Court and meeting the requirements of Section 3.3.

  - Parent Tract

The original tract prior to any division.

  - Daughter Tract

Any of the tracts created by divisions of a parent tract, including the remainder of the parent tract itself.

  - PRIVATE STREETS OR ROADS

Refer to any right-of-way not dedicated to the public and restricted to the use of certain property owners and their needs.

  - PUBLIC STREETS OR ROADS

Refer to any public right-of-way owned or controlled by a city, county, or state and maintained by the same for traffic use.  This definition does not include streets dedicated for public use and not accepted for maintenance by the aforementioned governmental entities.

  - UTILITY

Utility means a person, including a legal entity or political subdivision, that provides the services of an electric, gas or water and sewer utility.

  - UTILITY EASEMENT

Utility easement means an interest in land granted to the county, to the public generally, and/or to a private utility corporation, for installing utilities across, over or under the private or public land, together with the right to enter thereon with machinery and vehicles necessary for the maintenance.

  - VARIANCE

A permit that allows for a departure from the required standards of the Development Rules and regulations.  Variances are intended to resolve practical difficulties or unnecessary hardships that may result from the size, shape, or dimension of a site, location of existing structures, or geographic, topographic, or any other physical condition on the site or in the immediate vicinity.  Financial and/or economic hardships are not sufficient grounds for granting a variance.

Guarantee of Performance

No plat of any subdivision shall receive final approval unless the subdivider has complied or provided for compliance with the policies and procedures set forth in these regulations as they may be applicable. Until these policies and procedures have been complied with by the subdivider and the plat approved by the Commissioners Court as herein required, no septic system permit or development permit shall be issued by the Franklin County Environmental Services as to any property in an unrecorded subdivision developed subsequent to this Order.

- Platting Procedure

  - The preliminary and final platting procedure for subdivisions within Franklin County will be as follows:

    - Prior to any subdivision of land and any official submittal of a plat for review, the Owner shall set a meeting with the Precinct Commissioner. The Owner shall present a preliminary plat showing the street alignments and the lots. The Precinct Commissioner will provide general comments and requirements to the Owner. Based upon these comments and requirements, the Owner or his representative will submit four (4) copies of the revised preliminary plat of the subdivision to the Precinct Commissioner's office.

    - The Precinct Commissioner and/or his designee will review the plat for compliance with the Franklin County Subdivision Regulations.

3.1.3	The Precinct Commissioner and/or his designee will return his/her comments and recommendations to the Owner or his agent.

    - The Owner will address the comments and recommendations and set a meeting with the Precinct Commissioner to resolve the comments.

3.1.5  The Precinct Commissioner will request placement of the plat on the Commissioner's Court agenda for approval if all comments have been addressed. The Precinct Commissioner will request placement of a variance request on the Commissioners' Court agenda if it is requested by the Developer and deemed necessary by the Precinct Commissioner.

3.1.6 If the property to be platted lies within the extraterritorial jurisdiction (ETJ) of a city, this procedure will be accomplished simultaneously with the platting procedure of the city. The owner must obtain approval from the city before approval by the Franklin County Commissioners' Court.

Preliminary Plat

  - The preliminary plat shall be drawn to a scale of one hundred (100) feet to

- one (1) inch or larger and shall include the following:

3.2.1 Proposed name of the recorded subdivision, which shall not have the same spelling as or be pronounced similar to the name of any other subdivision located within Franklin County.

3.2.2  Acreage and number of lots within the subdivision.

3.2.3  A transmittal letter containing the name, address, telephone number, fax, and email for the subdivision owner.

3.2.4 Names of contiguous recorded subdivisions and the owners of contiguous parcels of unsubdivided land, and whether or not contiguous properties are platted.

3.2.5 Date of preparation, scale of plat and north arrow.

3.2.6 Vicinity sketch map that shall show the relation of the subdivision to well-known streets, railroads, and water courses in all directions to a distance of at least one (1) mile.

3.2.7 Topographical information shall include contour lines on a basis of five (5) vertical feet in terrain with a slope of two (2) percent or more, and on a basis of two (2) vertical feet in terrain with a slope of less than two (2) percent, which can be obtained/available from public sources. Topographical information is not required for the final plat.

3.2.8 Tangent lengths, centerline radii, names, and right-of-way dimensions for all proposed and existing roads and streets. Names of all proposed roads or streets shall not have the same spelling or be pronounced similar to the name of any other street located within Franklin County unless such proposed road or street is an extension of an existing road or street, in which case it shall be named the same as the existing road or streets.

3.2.9 Proposed easements, existing easements, and detention ponds.

3.2.10 Proposed approximate property line dimensions.

3.2.11 A statement whether the property is in the extraterritorial jurisdiction (ETJ) of a city or not.

3.2.12 The legal description of the land to be subdivided shall be sufficient for the requirements of title examination, including the current deed.

3.2.13 A Statement of Conformance in which the developer shall declare that the preliminary plat (including all supporting documentation) conforms to these regulations or the instances in which it does not comply with these rules shall be listed along with a reason for each such non-compliance and whether a variance might be requested.

3.2.14 Any restrictive covenants proposed to be imposed for the subdivision should be attached for reference.  It should be noted whether these are existing or proposed, and further, if proposed, at what time they will be recorded, prior to or subsequent to the recording of the final plat.

3.2.15 Designation of the water and sewer (if applicable) utility provider, if known, and the source of water intended to serve each of the lots.

3.2.16 Certification that all lots have been designed in compliance with the Rules and regulations of the Franklin County On-Site Sewage.

3.2.17 Provide letters signed by each utility company stating their plan for providing utility service within the proposed subdivision, e.g., electric, gas, telephone/cable/internet service, trash pickup, and water.

Final Plat

  - Since the county must approve or reject the submission within 30 days, it is the obligation of the owner or developer to submit information, provide notices, and schedule hearings in adequate time to permit proper review by the County. If this is not done, the submission will be rejected and a new application must be filed. Every final plat submission must include the following:

.

3.3.1   Owners and any lien holder's dedication, and restrictions, if any duly acknowledged in the manner required for the acknowledgement of deeds. The landowners shall dedicate the fee ownership in the land for street, drainage, and utility purposes. (See "Street or Road" definition, Section 2.2). Right-of-way easements for widening roadways or improving drainage must be accomplished by a plat note as found in Section C.1, placing the burden of maintaining the property upon the landowner until road or drainage improvements are actually constructed on the property.

3.3.2   A plat note was found in Appendix G(1)(2) stating that dedication of all public roadways and easements shall be accomplished free of liens except those liens stated in Section 5.2. Any required release of liens shall be provided to the Commissioners' Court.

3.3.3   Perimeter field notes certified by a Registered Surveyor.

3.3.4    All easements of record and visible and apparent that have a designated route shall be shown on the plat.  The owner and/or developer will be responsible for coordinating with all utility providers so that the location(s) of all public utility easements are shown on the final plat.

3.3.5   The plat shall have a statement indicating if all or any portion of the property falls within a flood hazard boundary area as identified on the most current Franklin County Flood Hazard Boundary Map

(FHBM) published by the Federal Emergency Management Agency.

3.3.6  A plat note was found in Appendix h for Commissioners' Court approval, including authorization for the County clerk to file the plat for record. These notes are to be located in the lower right-hand corner of the last sheet of the plat.

3.3.7  A statement of the total length of all streets in the proposed subdivision.

3.3.8  Approval of the incorporated city when the subdivision is within the extraterritorial jurisdiction (ETJ) of that city. The owner must obtain approval by the city before approval by Commissioners' Court.

3.3.9    A letter of serviceability from an entity or entities providing water service, or if no service is available within 300 feet of the subdivision, the Owner shall provide documentation prepared by an engineer registered to practice in this state, certifying that adequate ground water is available for the subdivision and that lots are suitable for private wells.

3.3.10   A person may not file for record or have recorded in the county clerk's office a plat or replat of a subdivision of real property unless the plat or replat has attached to it an original tax certificate from each taxing unit with jurisdiction of the real property indicating that no delinquent ad valorem taxes, including roll back taxes, are owed on the real property.

3.3.11 A statement from the owner acknowledging that it is the responsibility of the owner, not the County, to assure compliance with the provision of all applicable state, federal, and local laws and regulations relating to the environment. This shall be part of the owner's dedication statement on the face of the plat.

3.3.12  Certification by an engineer registered to practice in this state under his or her professional seal that all engineering for roads and drainage within the subdivision have been completed in compliance with these Regulations (including the Engineering Guidelines incorporated as Appendix B) and with all generally accepted engineering standards. The owner may defer this certification by meeting the security requirements of Section 5.2 or Section 5.3.

3.3.13  A signature block as found in Appendix q on the plat for approval by the Franklin County OSSF agent showing that they have examined the plat and that the plat is in compliance with the Franklin County Private Sewage Facility Regulations, Construction Standards for On-Site Sewage Facility Regulations (TCEQ). The Overall Site Plan for On-Site Sewage must be submitted  to the Franklin County OSSF agent a minimum of 45 days prior to submitting a Subdivision Permit Application. A representative of the OSSF Department, prior to final plat approval, MUST SIGN this signature block. This signature block must be incorporated into the plat during the draft stage.

3.3.14  It is the responsibility of the Owner to assure that the proposed name of the subdivision and all road and street names are not duplicated. The Owner shall check with ARK-TEX’s Counsel of Government (ARK-TEX COG) for verification.

  - Until the owner meets every subdivision requirement, no approval shall be granted by the Commissioners' Court with regard or concern to any subdivision.

  - At least seven days before the date of the Commissioners' Court's meeting for final plat approval, the owner shall submit to the county clerk the original plat and one identical copy. The final plat shall be drawn in permanent ink on reproducible film sheets twenty-four (24) inches wide and thirty-six (36) inches long, with margins of not less than one (1) inch on all sides. The plat shall be drawn at a scale of one hundred (100) feet to one (1) inch or larger. Where more than one sheet is necessary to accommodate the entire area, an index showing the entire subdivision at an appropriate scale shall be attached to the plat. The County Judge's approval and the County Clerk's approval shall be located on the last plat page on the lower right-hand corner with the County Clerk's approval being last.

Recorded Plat

3.5	Four (4) 18"x 24" black line copies of the Final Plat shall be presented to the County Clerk for recording as the Record Plat. The required size and proper filing fees as set out by the County Clerk's Fee Schedule. All writing and drawings on the Record Plat must be large enough to be easily legible.

FILING DEADLINES AND HEARINGS

3.6  	A completed Preliminary plat and application, with four copies, must be on file with the County Clerk thirty (30) days prior to the Preliminary Hearing. Upon receipt of a completed application and plat, the hearing on the final plat shall be 14 days after the Preliminary plat hearing but in no event shall it be over 60 days from the filing of the COMPLETED application.

3.7	Unless the preliminary plat approval is followed by a final plat approval within one year, the preliminary plat lapses and the subdivision must be resubmitted for approval.

3.8	The final plat must be recorded within six (6) months of approval by the Franklin County Commissioners' Court. The Commissioners' Court may grant a single six (6) month extension.

### 4. Re-subdivisions

4.1	Once the owner of an existing lot or lots in a legally platted subdivision has followed the procedure as found in Sections 3.1 and 3.3, the owner of an existing lot or lots in a legally platted subdivision may initiate a re­ subdivision by notifying the Precinct Commissioner. The Commissioner will place the plat on a Commissioners' Court agenda in order to set a public hearing on the proposed re-subdivision.

4.2	The person proposing the re-subdivision must publish notice of the hearing in a local newspaper of general circulation, at private expense. Notice of public hearing signs shall be placed next to all adjacent streets. The signs will be placed on 300-foot intervals. Whoever places the signs will be required to verify in writing at the public hearing that the signs remained in place during the previous 21 days. This will give other owners within the subdivision the opportunity to assert any legal claims against the proposed subdivision.

4.3	A proposed plat meeting the requirements of these Regulations must be submitted to the Court signed by each owner (including lien-holders) of the property to be re-subdivided. The owners of property elsewhere in the sub-division need not give specific written consent.

4.4	If the Court finds after the public hearing that the re-subdivision will affect no established legal rights, it will issue an order partially vacating the original plat and approving the plat of the re-subdivision. If the Court finds that the re-subdivision will affect established legal rights, it shall not approve the re-subdivision without the consent of all affected parties.

4.5     A public hearing will not be required if a public hearing for the proposed re-subdivision was held by a municipality.

### 5. Road and Drainage Construction

5.1	To protect the public interest, the Commissioners’ Court of Franklin County hereby decrees under the provision of Chapter 232, Local Government Code, that the owner of any tract of land that desires to obtain approval of a subdivision plat for recording a plat in the county records shall construct all streets, roads and drainage in said subdivision to the standards and specifications set forth by the Franklin County Commissioners' Court.

5.2	If the owner desires to have the plat placed on record before completion of

construction of the streets, roads and drainage, then the owner shall give a good and sufficient bond, cash, or letter of credit. This security must be payable to the County Judge, or his/her successors in office, of Franklin County Texas, the estimated cost of construction according to the calculations of an engineer registered to practice in this state. The security shalt be conditioned on the completion (in compliance with the road and drainage standards and specifications set forth by the Franklin County Commissioners' Court) of all the roads, streets and drainage shown on the plat, and such streets, roads and drainage are acceptable by the Commissioners' Court.

5.3	The developer may request partial reductions of the security requirement upon approval by the County Commissioners' Court, upon partial completion of construction of said road and/or drainage.

5.4	The owner shall submit construction plans for streets, roads and drainage and utilities within a platted subdivision to the County Commissioners' Court for approval prior to beginning construction. These plans shall show the location of all utilities and easements.

5.5	If landscaping and/or irrigation are proposed within the right-of-way, the owner shall create a body (homeowners association, neighborhood association, etc.) that will be responsible for the maintenance and liability of the landscaping and/or irrigation system. This body shall have assessment authority to ensure the proper funding for maintenance.

5.6		Once the construction has been completed and the Franklin County Road and Bridge Department has approved the construction, the Owner shall file with the County Clerk a set of "As Built" plans to be attached and     recorded with the original plat. These plans are to show the improvements as they were actually built. If the County determines that the plat contains flag lots that are a detriment to the public interest, welfare and/or safety, the County may require internal road construction at the sole discretion of the Commissioners' Court.

5.7	All postal mailboxes shall meet current U.S. Postal Service and Texas Department of Transportation standards.

6. Road Maintenance

6.0 	By accepting a subdivision plat for filing, the Commissioners' Court does not thereby accept the streets in the subdivision for ownership or maintenance by the County. The owner or owners of the platted lots are responsible for maintenance for all streets or roads within the subdivision unless the roads have been accepted by the Commissioners' Court.

6.1.1	The County will not accept a road for maintenance without the following pre-conditions:

6.1.2	Written certification from an engineer registered to practice in this

state that the road was constructed in accordance with the road and drainage standards and specifications set forth by the Franklin County Commissioners' Court in effect when the subdivision was legally platted (or has been upgraded to those standards). If the subdivision where the road is located was never platted, it must meet the current guidelines.

6.1.3	Written certification from an engineer registered to practice in this state that the road is currently in compliance with the applicable guidelines. The cost of any improvements, maintenance, or repairs required to meet that standard shall be borne by the developer or current landowners, not by the County.

6.2	The enforcement of plat restrictions is the responsibility of the developer and other owners in the subdivision.

6.3	The County will assume no responsibility for drainage ways in the subdivision, other than those running on or along the streets and roads. Maintenance and liability of landscaped areas within the right-of-way will be the responsibility of the developer, the neighborhood association or other owner entity.

7 Private Subdivisions

If a developer wishes to create a subdivision utilizing private roads, it must meet the following requirements:

7.1	The roads must meet all county road standards, except where specific variance has been granted by the Commissioners' Court for adequate cause in each case.

7.2	The subdivision plat and restrictions must contain a statement that Franklin County will never accept or maintain the roads unless they meet the county standards in effect on the date of acceptance.

7.3	The subdivision plat must contain a statement that the roads will be maintained in perpetuity by the owners in the subdivision, and must contain a mechanism for assessing the owners to produce adequate revenue for perpetual maintenance.

7.4	The plat must contain a requirement that every deed contain notice to the grantee that all roads and streets are private, that the owners will be perpetually liable for maintenance, that the county will never accept it for maintenance, and that the quality of the roads and streets affect access by public services such as police, fire and EMS.

7.5	All arterial and major collector streets required by Section B.2.1 must be dedicated to the public and constructed to county standards. Other streets will be dedicated to the homeowners’ association for the use of the property owners, their assigns and successors, and emergency response individuals.

7.6	The owner and/or developer will place a sign at the entrance of the

subdivision clearly stating that the roads are private roads.

7.7	The owner and/or developer will form a homeowners’ association with assessment authority. Membership in the association will be mandatory for each lot owner of the private subdivision. The association will be responsible for the maintenance of the roads in the subdivision.

7.8	Any owner that gates the entrances to the subdivision shall provide either a crash gate or a lock box and a letter of approval from all the affected emergency response agencies stating their approval of full-time access to the subdivision.

###### 8. Building Setbacks

  - The setback line on major highways and roads is set at 50 feet from the edge of the right-of-way.

  - The setback line on all public roads other than major highways and roads is set at a minimal of 25 feet from the edge of the right-of-way.

    - Variances

      - The Franklin County Commissioners' Court shall have the authority to grant variances from these Regulations when the public interest or the requirements of justice demand relaxing the strict requirements of rules.

      - Any person who wishes to receive a variance shall apply to the Precinct Commissioner, who will place it on the agenda of the Court and present it with his or her recommendation whether the variance should be granted.

      - The decision of the Court whether to grant or deny a variance is at its complete discretion, and will be final.

10. Penalties

  - Section 232.005 of the Texas Local Government Code provides for the

enforcement of the state subdivision laws and of these regulations.

  - A person commits an offense if the person knowingly or intentionally

Violates a requirement of these regulations, including the attached appendices; this offense is a Class B Misdemeanor punishable by fine and/or jail.

  - Under Texas law, a person may be jointly responsible as a party to an offense if the person (acting with intent to promote or assist the commission of the offense} solicits, encourages, directs, aids, or attempts to aid another person to commit the offense. Thus, a real estate agent or broker, a lender, an attorney, a surveyor, an engineer, a title insurer, or any other person who assists in violating these Regulations may also face criminal penalties.

  - Besides prosecuting a criminal complaint, the County Attorney may file a civil action in a court of competent jurisdiction to enjoin any violation or threatened violation of these Regulations, and to recover damages.

  - A tract that has been subdivided without compliance with these Requirements will be ineligible to obtain a permit for the construction or modifications of a Private Sewage Facility located on the tract.

    - Savings Clause

      - If any specific section of these regulations be deemed unenforceable and/or void, said determination will have no adverse effect on the enforceability of the Franklin County Subdivision Regulations as a whole.

Appendix A: Platting Guidelines

As a guide to the public in determining when it is necessary to file a plat and comply with these Regulations, the Commissioners' Court (as an incident of its power to enforce the subdivision laws and regulations under Section 232.003, Texas Local Government Code, and with the specific authority of Section 232.0015) has adopted the following policy guidelines stating when the division of an existing tract will be considered by the Court to be a subdivision requiring the filing of a plat by law, and thus requiring compliance with these regulations

A1. Form of Sale

A1.1	If a plat is required under these guidelines, it is immaterial that the sale of the daughter tracts is by contract, option, long-term lease (60 months or more), or lease-purchase, rather than by deed or that the daughter tracts are described by metes and bounds rather than lots and block.

### A2. Re-subdivision

A2.1		A plat is always required to divide a parent tract that is already located within a subdivision, even if one of the following exemptions would other otherwise apply. In addition, partial vacation of any existing subdivision plat will be required simultaneously to re-subdivide.

### A3. Size of Lots

A3.1	A plat is required if any daughter tract wilt be less than 10 acres in area. If any daughter tract is out of compliance with this requirement, the owner and/or developer must plat the entire subdivision (including any tracts that contain 10 acres or more).

A4. Adjacent Lots

A4.1		A plat is not required when the owner of two or more distinct adjacent tracts sells one or more of them, so long as all existing tracts remain intact. To be distinct the tracts must have a history of separate use.

A5. Family Grants

A5.1		A plat is not required when a person makes a conveyance of four or fewer tracts, each of which is sold, conveyed, given, or otherwise transferred to persons who are related to the owner within the third degree of blood relation (parent, child, grandparent, grandchild, sister, brother, great­ grandparent, great-grandchild, aunt, uncle, niece, nephew) or affinity (the spouse of anyone listed above, or so related to the owner's spouse) for their personal use; provided, however, that each daughter tract is either located on a public road or has access to such a road by a private easement. However, if the family members sell the land to a non-family member within two years, it will be presumed that the conveyance was not for personal use.

A6. Phased Subdivisions

A6.1	A plat is not required when daughter tracts are created solely for purposes of platting them as individual subdivisions in their own right. The owner and/or developer must submit to the Court the preliminary plat for the project as a whole before claiming this exception.

A6.2		A plat is not required when the owner retains title to a lot or portion of the land with direct access to a public road, and the other lot or portions are transferred to another person who will further subdivide the tract subject to the platting requirements herein.

###### A7. Property-Line Adjustments

A7.1•		A plat is not required when two adjacent landowners adjust or change the property lines that separate their respective tracts, so long as there is the same number of tracts and owners after the transaction as existed before it. This exemption applies whether the transaction requires an exchange of land by either owner, or only a transfer of land from one owner to the other, and whether the transaction takes the form of a sale or of an exchange in kind. However, any land added to a tract through such a transaction shall become an integral part of that tract, and may not be separately conveyed except in compliance with the subdivision laws. Each resulting tract shall, of course, be subject to the minimum lot size requirements of these Regulations and other applicable laws. This exception does not apply if the adjustment will change the boundary between two legally platted lots or subtract land from a legally platted subdivision.

A8. Partial Financing

A8.1		A plat is not required when a smaller tract is surveyed out of the parent tract solely for the purposes of obtaining financing for purchase or improvement of that part of the property, provided that possession and primary beneficial ownership of the entire parent tract are intended to remain unified, as provided that each tract has 50 feet of frontage on a public road.

A9. Foreclosure

A9.1 A plat is not required when a smaller tract is created by the legitimate foreclosure of a valid lien on a part of the parent tract. This provision does not exempt sham transactions or foreclosures staged to avoid the platting requirement.

###### A10. Partition

A10.1 A plat is not required if the property has been divided by the final decree of a court of record with appropriate jurisdiction.

A10.2 A plat is not required if all parts of the parent tract are transferred to persons who own an undivided interest in the original tract, and each tract created has direct access to a public road. A plat must be filed in accordance with these regulations before any further development of any part of the tract.

###### A11. Agriculture Use

A11.1 A plat is not required if every daughter tract is to be used primarily for agricultural use, as defined by Article VII, Section 1d, Texas Constitution, or for farm, ranch, wildlife management, or timber production use within the meaning of Article VIII, Section 1-d-1, Texas Constitution. If a tract of land ceases to be used primarily for these uses, the owner will be required to comply with these regulations on the same basis as any newly divided tract.

A12. Sale to Veterans

A12.1 A plat is not required if all daughter tracts are to be sold to veterans through the Veterans' Land Board Program.

###### A13. Sale by State Government

A13.1 A plat is not required if the owner of the land is the State of Texas, an agency, board or commission of the State of Texas, or a permanent school fund or other dedicated fund of the state, and the owner does not lay out any part of the tract for roads, parks, or other areas for the common use of two or more tracts or for the use of the public.

A14. Sale of Flood Plain by Government

A14.1 A plat is not required if the owner is a political subdivision of the State of Texas, the land is situated in a flood plain, and all lots are sold to adjoining landowners.

### A15. Documentation

A15.1 Any owner who claims to be entitled to any exclusion to platting set out in these guidelines must provide the following prior to the issuance of any development permits:

        - An affidavit claiming the exemption and setting out the detailed basis for exclusion from the platting requirements, subject to penalties for perjury.

        - A copy of the deeds or other instruments creating the daughter tracts referenced in the affidavit.

### Appendix B: Engineering Guidelines

###### B1. Lot Size

B1.1 A minimum lot size of one acre is required for all lots to be served by On-Site Sewage Facilities (OSSF).  Drainage easements and road widening easements are to be excluded from one-acre calculations.  All setback and separation distances as specified in 30 TAC 285.91(10) (Table X) must be met.

B2. Road Alignments

B2.1		Streets shall be laid out so as to align with existing streets in adjoining or nearby subdivisions, leaving the possibility of connecting such subdivisions with a minimum of road construction. No voids may be left within the subdivision with the intent of avoiding responsibility for construction roads or bridges.

B2.2		When two (2) or more arterial streets are in a subdivision, cross-streets at a maximum spacing of 1500 feet shall be provided to facilitate the movement of emergency vehicles such as fire trucks and ambulances. More frequent spacing for urban roads may be required, depending on traffic conditions and density.

#### B2.3	Dead-end streets shall have a cul-de-sac with a minimum right-of-way radius of 100 feet (Minimum base radius to be 44 feet) with "Dead End" or "No Outlet" street signs, at private expense, placed over the street name sign.

B2.4	The County may require an internal street system that minimizes road cuts to existing County or other public roadways.

### B3. Minimum Road Requirements

B3.1		As of the date of the submission of the plat application, the minimum road requirements of a subdivision shall be of the same standard as those imposed upon Franklin County concerning the construction of streets and roads with a similar amount of traffic.

B4. Street Names and Markers

B4.1		All streets and roads to be dedicated to the public within a subdivision shall be named. The street names shall be displayed on standard intersection street markers erected by the owners at private expense at each street intersection. All houses shall be numbered as approved by ATCOG.

B4.2		Traffic control signs (such as stop, yield, and speed limit signs) as approved by the Commissioners' Court shall be installed, at private expense, by the owner and/or developer of the subdivision, at all intersections. Other traffic control signs shall be installed to indicate any unusual traffic or road hazard, or conditions that may exist. All traffic control devices shall be placed in compliance with the current standards of the Texas Department of Transportation and all construction costs shall be included in the security. The placement of these signs shall be shown in the construction plans.

B4.3		The speed limit shall be set by the Franklin County Commissioners' Court upon recommendation of the Precinct Commissioner.

B4.4	The streets or roads in any subdivision will not be accepted for final maintenance by the Commissioners' Court unless all of the aforesaid requirements and conditions regarding street names, street signs, and traffic control signs have been complied with.

B4.5   A street name sign shall be placed, at the owner's expense, at the end of any street that is proposed to be extended at some time in the future. The sign will state the following: "Future Extension of (name of street)."

Appendix C: Plat Notes

###### C1a Road Widening Easements

C1.1 Right-of-way easements for widening roadways or improving drainage shall be maintained by the landowner until road or drainage improvements are actually constructed on the property. The County has the right at any time to take possession of any road widening easement for the construction, improvement, or maintenance of the adjacent road.

###### C2. Owner's Responsibility

C2.1		In approving the plat the Commissioners' Court of Franklin County, Texas, it is understood that the building of all streets, roads, and other public thoroughfares and any bridges or culverts necessary to be constructed or placed in the subdivision is the responsibility of the owners of the tract of land covered by this plat in accordance with the plan and specifications prescribed by the Commissioner's Court of Franklin County, Texas. Said Commissioners' Court assumes no obligation to build any of the streets, roads, or other public thoroughfares shown on this plat or to construct any of the bridges or drainage improvements in connection therewith. The County will assume no responsibility for drainage ways or easements in the subdivision, other than those draining or protecting the road system and streets.

C2.2		The County assumes no responsibility for the accuracy of representations by other parties in this plat. It is further understood that the owners of the tract of land covered by this plat must install at their own private expense all traffic control devices and signage that may be required before the

streets in the subdivision are finally accepted for maintenance by the County.

Appendix D: Application for Land Subdivision

DATE RECEIVED: _

CHECK ONE:______ Preliminary Plat  _______ Final Plat ________ Replat ______ Amended _______ Cancellation_____

PROPOSED SUBDIVISION NAME: 						______

UNIT NO.

LOCATION DESCRIPTION/NEAREST COUNTY ROAD: 							_________________________________________________

ACREAGE: 			 	NO. OF LOTS: EXISTING

PROPOSED 			___________________________________________

REASONS FOR PLATTING/REPLATTING: 								______________________________________________________

OWNER/APPLICANT:

(IF APPLICANT IS PERSON OTHER THAN OWNER, A LETTER OF AUTHORIZATION FROM THE OWNER IS REQUIRED)

ADDRESS:

TEL: 				 FAX: 				 MOBILE:

EMAIL:

LICENSED ENGINEER/SURVEYOR:

ADDRESS:

TEL: 				 FAX: 				 MOBILE:

EMAIL:

LIST ANY VARIANCES REQUESTED:

REASON FOR VARIANCE:									___________________________________________________________	___________________________________________________________

PRESENT USE OF PROPERTY:

INTENDED USE OF LOTS (check all that apply): 	 Residential (Single Family)

___ Residential (Multi-Family)  ____ Other (Specify) 	_______

IS THE PROPERTY LOCATED WITHIN CITY ETJ: 		 YES		 NO

(IF YES: NAME OF CITY):

IS ANY PART OF THE PROPERTY IN A FLOOD PLAIN?___________________

WATER SUPPLY:

ELECTRIC SERVICE:

SEWAGE DISPOSAL:

GAS SERVICE:

Is the property subject to any liens, encumbrances, or judgments?_   Yes 	__ No

If so, give details. (Provide a separate sheet if needed.)   ______________________________________________________________________________________________________________________________________________________________________________________________________

Permission from any lien holders and/or removal of any encumbrances or judgments will be necessary prior to filing of said plat with the County Clerk's Office.

See platting requirements. All necessary documents to reflect compliance must be complete before the application will be deemed complete.

I agree to comply with all platting and subdivision requirements of Franklin County, Texas. I understand that the plat will NOT be forwarded to the Commissioners' Court unless all documentation is satisfactorily filed with the County Clerk's Office.

Signature of Owner/Applicant				Print Name and Title

Date

Appendix E:

Subdivision Platting Checklist-First Reading (Preliminary)

Yes		No	  	N/A

Name of Proposed Subdivision

Name and address of Sub-divider.

Volume, page and reference names of adjoining owners.

Volume, page and reference land use of adjoining owners.

Master Development Plan (if subdivision is a portion of a larger Tract).

Location map.

Scale (not smaller than 1"=100'). If parent tract is larger than 320 acres, scale may be 1"=l,000' with proposed plat 1"=100'.

North Directional Arrow

Contour information (rivers, creeks, bluffs, etc.) (no greater than 2’ intervals)

Major topographic features.

Total acreage in subdivision.

Total number of lots in subdivision.

Typical lot dimensions.

Land use of lots, parks, greenbelts.

Total length of roads.

Width of right-of-way.

Special flood hazard areas/note.

Road maintenance (County/Home Owners Assn.)

Approval by TxDOT or County for driveway entrance(s).

Yes		No	  	N/A

Location of wells -water, gas, & oil, where applicable & unused capped statement.

Plat Filing Fees paid. (receipt from County Clerk required)

On-Site Sewage Facility Inspector’s Approval.

_____		_____		______		Acknowledgement of Rural Addressing / Signage.

Water Availability Study.

Tax Certificates and rollback receipts.

Signature of Reviewer					Date of Review

ADDITIONAL REQUIREMENTS:

ALL ITEMS ON THIS CHECKLIST MUST BE IN THE HANDS OF THE COUNTY CLERK'S OFFICE NO LESS THAN THIRTY (30) DAYS PRIOR TO THE COMMISSIONERS' COURT HEARING DATE.

Appendix F: Subdivision Platting Checklist

Second (Final) Reading

Yes		No	  	N/A

All information required for preliminary plat.

Lot and block numbers.

Street names must be pre-approved by     9-1-1 Coordinator.

Acreage of each lot or parcel.

Name and address of Surveyor/Engineer.

Location and size of drainage structures.

Location, size, and proposed use of easements.

Incorporated City's Boundary/ETJ Note.

Servicing Utilities Note.

Certification from a licensed professional engineer regarding utilities.

Restrictive covenants.

Tax certificates and rollback receipts if

required.

Home Owners' Association Incorporation articles and by-laws.

Construction plans of roads and drainage improvements.

Receipt showing payment of Final plat fees.

Sign-off for TxDOT road access, if applicable.

Appendix G(1) - Certificate of Dedication by Owner (when owner is an individual)

Appendix G(2) - Certificate of Dedication by Owner (when owner is a corporation)

Appendix H - Certificate of Recording (if applicable)

Appendix I - Water Supply Certificate

Appendix J -Certificate of Surveyor

Appendix K - Certificate of Engineer

Appendix L -Certificate of Road Maintenance (when roads are to be retained as private roads)

Appendix M - Certificate of County Approval (not applicable until the Court hears request to assume maintenance of roads)

Appendix N - Franklin County Permit to Construct Access Driveway Facilities on County Road Right-of-Way

Appendix O - Lienholder's Acknowledgement

Yes		No	  	N/A

Appendix P - Revision to Plat

Appendix Q - On-Site Sewage Facility Inspector's Approval

Appendix R - Utility Line Installation Permit

Signature of Reviewer					Date of Review

ADDITIONAL REQUIREMENTS:

ALL ITEMS ON THIS CHECKLIST MUST BE IN THE HANDS OF THE COUNTY CLERK'S OFFICE NO LESS THAN THIRTY (30) DAYS PRIOR TO THE COMMISSIONERS' COURT HEARING DATE.

Appendix G(1):

CERTIFICATE OF DEDICATION BY OWNER

(When Owner is an Individual)

THE STATE OF TEXAS		§

§

COUNTY OF FRANKLIN		§

KNOW ALL MEN BY THESE PRESENT, that I, 			 , owner of 		 acres of land out of the 				 Survey, Franklin County, Texas as conveyed to me by deed dated _________ , and recorded in Volume ___ , Page ___ , Real Property Records of Franklin County, Texas, DO HEREBY SUBDIVIDE ___________ acres of land out of said Survey,

(Note: if the subdivision lies in more than one survey, determine an acreage in each survey and repeat for each original survey within the subdivision)

to be known as the ____________ Subdivision, in accordance with the plat shown hereon, subject to any and all easements or restrictions heretofore granted, and do hereby dedicate to the public (or "owners of the property shown hereon for private streets”) the use of the streets and easements shown hereon.

WITNESS MY HAND, this the __ day of _______ , A.D., 20____.

Owner’s Name

THE STATE OF TEXAS		§

§

COUNTY OF FRANKLIN		§

BEFORE ME, the undersigned authority, on this day personally appeared, ___________________________________, known by me to be the person whose name is subscribed to the foregoing instrument and acknowledged to me that (s)he executed the same for the purposes and consideration of therein stated.

GIVEN UNDER MY HAND AND SEAL OF OFFICE this the _____ day of _____________, A.D., 20_____.

Notary Public, State of Texas

Appendix G(2):

CERTIFICATE OF DEDICATION BY OWNER

(When Owner is a Corporation)

THE STATE OF TEXAS		§

§

COUNTY OF FRANKLIN		§

KNOW ALL MEN BY THESE PRESENT, that ________________________, a corporation organized and existing under the laws of the State of Texas, with home address at ________________________________________________________ and owners of __________ acres of land out of the ___________________ Survey, in Franklin County, Texas, as conveyed to it by deed dated ___________________ and recorded in Volume ___ , Page ___ , Real Property Records of Franklin County, DOES HEREBY SUBDIVIDE ___ acres of land out of said Survey,

(Note: if the subdivision lies in more than one survey, determine the acreage in each survey and repeat for each original survey within the subdivision)

to be known as the ____________ Subdivision, in accordance with the plat shown hereon, subject to any and all easements or restrictions heretofore granted, and does hereby dedicate to the public (or "owners of the property shown hereon" for private streets) the use of the streets and easements shown hereon.

IN WITNESS WHEREOF the said ___________ has caused these present to be executed by its _____________ , thereunto duly authorized, this the ___ day of _____ _, A.O., 20 __ .

ATTEST:

(Name, Title)						(Name, Title)

THE STATE OF TEXAS		§

§

COUNTY OF FRANKLIN		§

BEFORE ME, the undersigned authority, on this day personally appeared ____________ known to me to be the person whose name is subscribed to the foregoing instrument as an officer of __________ and acknowledged to me that the foregoing was executed in such capacity as the act of said corporation for the purposes and considerations therein stated.

GIVEN UNDER MY HAND AND SEAL OF OFFICE this the _____ day of _____________, A.D., 20_____.

Notary Public, State of Texas

Appendix H:

CERTIFICATE OF COUNTY APPROVAL OF PLAT FOR FILING PURPOSES ONLY

THE STATE OF TEXAS		§

§

COUNTY OF FRANKLIN		§

I, _________ , County Clerk of Franklin County, Texas, do hereby certify that on the ______ day of ___________________ , A.D., 20___ , the Commissioners' Court of Franklin County, Texas, passed an Order authorizing the filing for record purposes only this Plat, and said Order has been duly entered in the minutes or the said Court in Book ______ , Page ______.

WITNESS MY HAND AND SEAL OF OFFICE this ______day of ______________, A.D., 20______.

, County Clerk

Franklin County, Texas

, County Judge

Franklin County, Texas

APPENDIX I:

WATER SUPPLY CERTIFCATE

"No structure in this subdivision shall be occupied until connected to either: an individual water well, the location of which has been approved by the Northeast Texas Municipal Water District, a TCEQ approved public water supply system (described below), or other domestic water supply subject to approval by the Franklin County Commissioners' Court."

Northeast Texas Municipal Water 		Date

District

Name of Public Water Supply System		Date

Signature & Title of Authorized Agent

Other Proposed Domestic Water Supply (Please Specify):

APPENDIX J:

CERTFICATE OF SURVEYOR

THE STATE OF TEXAS		§

§

COUNTY OF FRANKLIN		§

KNOW ALL MEN BY THESE PRESENT, that I, the undersigned, a Registered Professional Land /Licensed State Land Surveyor in the State of Texas, do hereby certify that this Plat complies with the survey related requirements of the Franklin County Subdivision Regulations and I further certify that this plat is true and correctly made and is prepared from an actual survey of the property made under my supervision on the ground and that the corner monuments were properly placed under my supervision.

Registered Professional/Licensed		Date

State Land Surveyor

License No.

APPENDIX K:

CERTIFICATE OF ENGINEER

THE STATE OF TEXAS		§

§

COUNTY OF FRANKLIN		§

KNOW ALL MEN BY THESE PRESENTS, that I, the undersigned, a Licensed Professional Engineer in the State of Texas, hereby certify that this plat complies with the engineering related requirements of the Franklin County Subdivision Regulations.

Licensed Professional Engineer		Date

License No.

(Seal)

APPENDIX L:

CERTIFICATE OF ROAD MAINTENANCE

(When roads are to be maintained as Private Roads)

"In approving this plat by the Commissioners Court of Franklin County, Texas, it is understood that all roads shown hereon are private roads and shall remain the property of the Sub-divider and/or subsequent owners of the property. The construction, repair, and maintenance of these roads and any associated drainage improvements will be the responsibility of the Sub­divider and/or subsequent owners of the subdivision and will not be the responsibility of Franklin County."

Sub-divider or Representative				Date

APPENDIX M:

CERTIFICATE OF ROAD MAINTENANCE

(When roads are to be dedicated to Franklin County for maintenance)

"In approving this plat by the Commissioners' Court of Franklin County, Texas, it is understood that all roads shown hereon are private roads and shall remain the property of the Sub-divider and/or subsequent owners of the property until such time as the Commissioners' Court approves the dedication of the roads to the County for maintenance by way of a Warranty Deed. Acceptance of this plat does not constitute acceptance of the roads shown hereon by Franklin County."

Sub-divider or Representative				Date

APPENDIX N:

PERMIT TO CONSTRUCT WITHIN COUNTY ROAD RIGHT-OF-WAY

APPLICANT:

COUNTY ROAD NAME:

ADDRESS: 							 PERMIT #:

PHONE:

I, the Precinct Commissioner for Franklin County Texas authorize ______________ __________________, hereinafter called the Grantee, to (re)construct an access driveway on the County road right-of way abutting County Road ______________ in Franklin County, Texas, located at 							;

SUBJECT TO THE FOLLOWING:

1.	The Grantee is responsible for the culvert costs and installation.

2.	Design of facilities shall be as shown on the sketch on page 2.

3.	All construction and materials shall be subject to inspection and approval by the County.

4.	The County reserves the right to require any changes, maintenance, or repairs as may be necessary to provide protection of life or property on or adjacent to the County road. Changes in design will be made only with approval of the County.

5.	The Grantee shall hold harmless the County and its duly appointed agents and employees against any action for personal injury or property damage sustained by reason of the exercise of this permit.

6.	The Grantee shall not erect any sign on, or extending over, any portion of the County road right-of-way.

7.	Vehicle service fixtures such as fuel pumps, fuel tanks, vendor stands, etc., shall be located at least 12 (twelve) feet from the right-of-way line to insure that vehicles being serviced from these fixtures will be off the County road.

8.	Entrances must be constructed in such a way as to keep obstructions from being present in the right-of-way.

9.	Mail boxes must be mounted on break away stands and be located so that boxes may be serviced and used from the pavement.

10.	This permit will become null and void if the above referenced driveway facilities are not constructed within six (6) months from the issuance date of this permit.

11.	The Grantee will contact the Court's representative ___________________ at telephone number _________________________ at least twenty-four (24) hours prior to beginning construction which is authorized by this permit.

DATE OF ISSUANCE:

PRECINCT COMMISSIONER:

The undersigned hereby agrees to comply with the terms and conditions set forth in this permit for construction of an access driveway on the County road right-of-way.

PRINTED NAME:

SIGNATURE:

DATE:

**********************************************************************************************

SKETCH OF INSTALLATION

APPENDIX O:

LIENHOLDER’S ACKNOWLEDGEMENT

I (We), 				 [name of lienholder(s)], owner(s) and holder(s) of a lien(s) against the property described within the Revision to Plat, said lien(s) being evidenced by instrument of record in Volume ___ , Page ___ , of the Real Property Records of Franklin County, Texas, do hereby in all things subordinate to said Revision of Plat said lien(s), and I (we) hereby confirm that I am (we are) the present owner(s) of said lien(s) and have not assigned the same nor any part thereof.

Signature of Lienholder(s)

Printed Name(s)

THE STATE OF TEXAS		§

§

COUNTY OF FRANKLIN		§

SWORN AND SUBRCRIBED before me by _				_____	 on the ______ day of _____________, A.D., 20_____.

Notary Public, State of Texas

APPENDIX P:

REVISION TO PLAT

Name of Subdivision: __________________________

Recorded in Volume ______, Page _______ of the Plat Records of Franklin County, Texas

Commissioner Precinct No.: __________________________

Owner: __________________________

Owner’s Mailing Address: __________________________

Owner’s Phone Number(s): __________________________

Lots or Tracts to be Revised (include Unit, Section or Phase# if applicable):

Resulting Lot Number to be Known As:

Lienholder: ___ Yes ___ No

If yes, Name of Lien holder: __________________________

(attach Lienholder’s Acknowledgment-Appendix O)

IF REVISED PLAT INCLUDES ANY CHANGES TO AN EXISTING UTILITY EASEMENT, RELEASE OF SAID EASEMENTS BY THE UTILITY PROVIDERS IS REQUIRED BEFORE APPROVAL OR FILING OF SAID PLAT.

The signature affixed below will certify that the owner of the described property does hereby request to revise the plat of the property. The owner certifies that any and all lienholders have acknowledged this revision as per the attached Lien holder's Acknowledgement, if applicable.

Owner’s Signature

Printed Name

THE STATE OF TEXAS		§

§

COUNTY OF FRANKLIN		§

SWORN AND SUBSCRIBED before me by __________ on the ______ day of _____________, A.D., 20_____.

Notary Public, State of Texas

APPROVED BY COMMISSIONER’S COURT ON THE __________ DAY OF ________________________, A.D., 20________.

Franklin County Judge

ATTEST:

Franklin County Clerk

APPENDIX Q:

CERTIFICATE OF ON-SITE SEWAGE FACILITY INSPECTOR’S APPROVAL

THE STATE OF TEXAS		§

§

COUNTY OF FRANKLIN		§

KNOW ALL MEN BY THESE PRESENTS, that I, the undersigned, a Licensed On-Site Sewage Facility Inspector in the State of Texas, hereby certify that I have inspected the On-Site Sewage Facilities for this plat, and the same complies with the related requirements of the Franklin County Subdivision Regulations and the TCEQ.

On-Site Inspector							Date

License No.

(Seal)

[NOTE: The inspector may be required to be present for questioning at the presentation of the plat to the Commissioners' Court.]

APPENDIX R:

NOTICE OF PROPOSED UTILITY LINE INSTALLATION FRANKLIN COUNTY RIGHT OF WAY AND PERMIT

TO:	FRANKLIN COUNTY ROAD SUPERVISOR:

Address

Formal Notice is hereby given that 					 proposes to place a ________ line within the right of way of County Road __________ as follows: (list location, length, general design, etc.)

Installation will begin on or after the _____ day of _______________________, 20_______.

The line will be constructed and maintained on the road right-of-way as directed by Franklin County Commissioner Precinct No. ________.

The location and description of the proposed line is more fully shown on the attached drawings.

Applicant will insure that traffic control measures complying with applicable portions of the Texas Manual of Uniform Traffic Control Devices will be installed and maintained during the installation.

If the proposed installation is a parallel installation, the installation shall be located ___ feet within the edge of the right-of-way and at least ____ feet in depth, unless otherwise approved by the County.

___ The installation shall not damage any portion of the road and adequate provisions must be made to cause minimum inconvenience to traffic and adjacent property owners during installation.

OR

___ The installation shall damage a portion of the road. Applicant will return the road to its pre-installation condition at Applicant's expense within __ days of installation. During installation adequate provisions must be made to cause minimum inconvenience to traffic and adjacent property owners during installation.

Applicant agrees that any damages sustained to the line installed under this proposal as a result of road construction and/or maintenance, including but not limited to mowing, ditch cleaning, culvert repair or replacement, roadway excavation, and base work shall be the sole burden and expense of the owner of the utility line.

Applicant agrees to give Franklin County Precinct Commissioner fifteen (15) days prior notice of any routine or periodic maintenance which requires interruption of traffic and pruning of trees within the road right-of way. County may provide specifications for the extent and methods governing trimming, cropping, tree balance, type of cuts, painting cuts, and clean up.

Applicant agrees that Franklin County does not purport to grant any right, claim, title, or easement in or upon this road, and Applicant further agrees that Franklin County may require owner to relocate line, subject to provisions of governing laws, upon the giving of __ days written notice.

In the event Applicant fails to comply with any of the requirements as set forth above, Franklin County may take such action as it deems appropriate to compel compliance

Additional Special Provisions:

By signing the below, I certify that I am Applicant or am authorized to represent Applicant and that Applicant agrees to be bound by the provisions of the Notice and Permit.

APPLICANT:

Name:

Authorized Agent:

Address:

Phone:

FRANKLIN COUNTY:

Precinct Commissioner:

Address:

Phone: